3. The Lane Keeping Application

This project implements a robust algorithm in which the LEGO Mindstorms EV3 follows a trajectory on a white surface that is bordered with the help of two black lines between which the robot is situated, thus keeping itself in a lane.

Previous diploma projects including Mindstorms EV3 included the task of following a single black line. The purpose of the robot was to make correction turns when the sensor attached to it began to read another value other than the one of the black line. With the help of proportional, integral and derivative gains, the turns are adjusted from gentle turns to harsh ones.

So, unlike traditional robots that follow a single line this project focuses on a robot that uses two color sensors to balance itself inside the white “road” that is contrasted with the two black edges. It utilizes a custom controller combined with an emergency safety net to achieve a reliable and smooth navigation.

The robot is evaluating continuously its physical position by computing how far it has drifted from the ideal center and applying corrective steering to maintain its trajectory. In doing so, the robot uses two color sensors on each side of its front which adjust it from a disturbed state to an ideal state.

The Ideal State, described in the code by the variable setpoint, is the state in which the robot is perfectly centered, and both sensors read the exact reflection value from the white, meaning the difference is 0.

The Disturbed State is the state where the robot, by drifting left or right, moves over the black boundary line. This causes the sensor’s value on the black line to drop while the other sensor remains on white, so it keeps its value. In this situation, the difference between the two sensors becomes non-zero and it needs to be fed to control algorithms to be corrected.

This image provides a good example of a path the robot navigates while maintaining itself bounded by the black lines. The trajectory differs from straight, curved and cornered, covering most possibilities.

The architecture of this project provides an Object-Oriented, SOLID-compliant structure. So, there is a dedicated method initializeConfiguration(task, algorithm) where all necessary variables are initialized for this type of application:

The Left and Right Color Sensors are connected to the physical ports of the robot. The Black Threshold represents value 6 where the sensors are above the black line, while Crossing Threshold represents the value 7 where the sensors are very close to the black line. The Min Difference being 3 represents the tolerance zone where the sensors are above white surface, but they don’t read the exact same value (values close to 0 means dark and values close to 100 means bright). This allows the robot to continue straight, ignoring shadows on the white surface. Base Speed is the forward velocity of the robot being perfectly centered. It is 50 mm/s since the track isn’t long and can be easily followed. Turning Speed being 25 mm/s is the reduced forward velocity of the robot during a regulation to prevent the robot from overshooting its turns. Emergency Speed (20 mm/s) and Turn Speed (100 deg/s) are highly restrictive forward speed, respectively extreme rotational speed, used exclusively to aggressively pivot the robot away from a lane exit. The Stop Confirm Count is the number of times the robot detects the value of black on both sensors before it stops. Due to false readings caused by shadows or oversteering in narrow corridors, this variable has been introduced. After many runs of the robot when there were no more false readings, the value of times it encounters black is set to 1. The Log Interval is the number of seconds to pass to print what the robot has recorded and what action did it took. It is 1 for a more detailed log.

The Lane Keeping application’s functionality is described in the LaneKeepingController class, which implements the Controller interface. In this class there are defined the methods which contribute to the desired behavior of the robot when it tries to follow the lane. It firstly initializes the attributes in the constructor, retrieves the values from the sensors, prints the header of the log, decides what action needs to be taken and finally the run method which starts the robot to move.

The constructor takes the inherited attributes from the super abstract class Controller and along with the more specific attributes taken from the configuration dictionary. These attributes initialize the state of the LaneKeepingController object.

This method simply reads the values of the light reflected on the surface they are currently on and returns them.

This method prints the header of the log to be able to compare the results and see how the thresholds affect the decision-making process after the run.

This method implements a safety layered architecture with three priority levels: emergency override, admitted tolerance and algorithm regulation. Based on this architecture, the method decides what action needs to be taken based on the values read by the sensors and the difference between them.

The first case treats the moment when the left sensor reads a value close to the black and the right sensor reads a larger value. This means that the robot is crossing the black line on the left side, so the method returns the emergency speed and the emergency turn speed with positive value, meaning turning right.

The second case is opposite to the first as it treats the moment when the right sensor reads a value close to the black and the left sensor reads a larger value. This means that the robot is crossing the black line on the right side, so the method returns the emergency speed and the emergency turn speed with negative value, meaning turning left.

The third case treats the difference with the values of the sensors being smaller or equal than the minimum difference accepted to let the robot continue straight ahead, meaning that the robot is on a bright surface and the method returns the normal speed and the speed of turning is 0 to move without deviation.

In the fourth case the disturbance and turn rate are taken from the results done by the compute() method of the selected algorithm and then sent to the PC. The method finally returns the turning speed and the turn rate (the output of the regulation) meaning that the robot is regulated by the selected algorithm.

The final run() method is the main execution loop. It combines four phases of the control cycle: sense, decide, actuate and wait. It includes the previous three methods to execute the task. Firstly, it identifies the algorithm by checking the instance of the algorithm class and attributes it to the algorithm variable. It prints the header according to the algorithm and the constants and resets the number of times both sensors crossed the black line and the time.

The continuous loop begins with the computation of the change in time by subtracting the current time (obtained from the now() method) with the last time and updating the last time. The left and right values are assigned with the values read by the sensors and then, based on the log interval, which in this case is 1, is printed at each multiple of it the step plus what values the sensors read. If the algorithm is Absolute Fuzzy, the rules are also printed.

With the help of the Boolean variables left on black and right on black, the condition for the robot to stop is created by having both values of the sensors smaller than the threshold value which indicates a very dark surface. In the event of false readings, the number of times both sensors were on the black line increases until it reaches the confirmation count to stop, otherwise it reduces the speed and waits to see if indeed is on a black line or not.

If the conditions fail, the difference is computed and the speed with the turn rate are computed from the action it needs to be decided based on the left value, right value, difference and the change in time. The returned speed can’t get lower than 10 and the robot is driving based on the final speed and turn rate. The final wait is used to ensure that the loop frequency remains consistent regardless of which branch was taken.

The except part of the try-except black where the program is interrupted by the user. The robot stops and the stream closes.

3.1 PID Algorithm

One possible option for the robot to succeed in following the lane is with the help of a PID controller. This algorithm that implements this type of controller uses Comparative Control, where the disturbance of the two color sensors, is computed at each step in order to be given as input to the controller so that it can generate the turn rate so that the disturbance is minimized.

In the first part of the definition of the class, the constructor initializes the constants which will be computed to determine the final turn rate, as well as the interval ends for it and the integral. The previous disturbance and the integral are set to 0.

The compute() method takes the setPoint which is usually 0, indicating perfect balance between sensors, the measuredPoint which is the actual difference between sensors and the change in time. The disturbance is the result of ideal minus actual. The final turn rate output is the sum of the three PID terms made of the imbalance and their gains:

Turn Rate = (KP × Imbalance) + (KI × Imbalance dt) + (KD × Imbalance d/dt)

3.1.1 Proportional Gain (KP)

It symbolizes the present as it only considers the current value of the Disturbance. As the main steering force, its effect on the robot is that the further the robot is from the center, the harder it turns. In other words, if the robot is sluggish and goes off the track on curves, increase the KP. If the robot shakes violently back and forth, decrease the KP. In the compute() method the proportional term is the product of the disturbance and the gain.

3.1.2 Integral Gain (KI)

It represents the memory since it accumulates the Disturbance over time. If the robot has a mechanical drift, for example a slightly weaker motor, or is on a long, sweeping curve, KP alone might leave the robot off-center while KI builds slowly steering power until the robot is perfectly centered. Usually this constant is very small, less than 0.1. Otherwise, it creates Integral Windup, meaning that the robot will overreact drastically and swing off the lane. In the compute() method the integral term is the product of gain and the disturbance multiplied with the change in time, constrained by the integral interval in order to prevent the integral from growing infinitely large.

3.1.3 Derivative Gain (KD)

This constant is the dampener as it computes how fast the Disturbance is changing. When the robot steers back toward the center of the lane, KD acts as a brake. It reduces the steering power right before the robot reaches the center, preventing it from overshooting into the other black line. A higher KD will tighten the path so that the robot will avoid crawling widely as a snake down the lane. In the compute() method the derivative term is the product of the gain and the derivative of the disturbance with respect to the time.

The output final turn rate is comprised of all three PID terms and restricted to the output interval so that the motors of the robot don’t receive impossible commands.

The previous disturbance is updated with the current one and the method returns the disturbance and the output. The PIDAlgorithm class also has a reset method to clear the memory of the controller by setting the integral and the previous disturbance to 0.

3.2 Fuzzy PI Algorithm

This algorithm stands apart from a rigid mathematical model as the PID controller by acting as a human driver who adjusts the steering power based on how off the center the robot is. A Fuzzy PI controller changes the constants KP and KI depending on the state of the robot using Fuzzy Logic.

In standard programming the logic is binary, where conditions are either true (1) or false (0). Fuzzy Logic introduces degrees of truth, allowing the robot to categorize a disturbance slightly off-center or very far off-center. By translating the crisp value of the Disturbance into fuzzy sets, the algorithm adjusts dynamically the proportional and integral gains to generate the final turn rate.

The FuzzyPIAlgorithm class implements the fuzzy PI algorithm following the three stages of a fuzzy control system: fuzzification, fuzzy inference and defuzzification. The fuzzification is the conversion of a crisp numerical value into a fuzzy set. The fuzzy inference is the approximate reasoning to formulate a mapping from premise to conclusion. The defuzzification is the reverse conversion of a fuzzy set into a crisp value. The constructor of the fuzzy PI algorithm initializes the base gains KP and KI which will be computed to determine the final turn rate, the fuzzy set boundaries S=3, M=6, L=10 describing the disturbance, the rule tuples that encode the gain multipliers for each fuzzy set.

3.2.1 Fuzzification

The fuzzifyDisturbance() method takes the crisp value of the disturbance and computes the membership degree for each fuzzy set: small S, medium M and large L. The µS, µM, µL, representing the membership degrees, are values between 0 and 1 and describe how much the input resembles each set simultaneously.

The three fuzzy sets are defined by triangular and trapezoidal membership functions. They are delimited by the boundary values from the configuration: S=3.0, M=6.0 and L=10.0. These boundaries determine where one fuzzy set begins and another ends, with overlapping regions where the disturbance partially belongs to two sets at the same time.  It checks for each fuzzy set because there can be more than one non-zero membership value, and that’s why the method returns all three membership values.

The picture above presents graphically the intervals where the disturbance can be situated and what membership value can have, describing a triangular/trapezoidal transition. The disturbance grows or is reduced resulting in smaller membership function for the initial set and a higher membership function for the next set.

For small, a trapezoidal membership function, has full membership (µS = 1.0) when             |d| ≤ S. Then it decreases linearly from 1 to 0 as the disturbance increases from S to M. Zero membership (µS = 0.0) when |d| ≥ M.

For medium, a triangular membership function, has zero membership (µM = 0.0) when     |d| ≤ S or |d| ≥ L. It increases linearly from 0 to 1 as the disturbance increases from S to M until it reaches its peak when the disturbance is exactly M and has full membership function (µM = 1.0). Then it decreases linearly from 1 to 0 as the disturbance increases from M to L.

For large, a trapezoidal membership function, has zero membership (µL = 1.0) when           |d| ≤ M. Then it increases linearly from 0 to 1 as the disturbance increases from M to L and reaches full membership (µL = 1.0) for any disturbance greater or equal to L.

These membership functions have the property that at any value of the disturbance the sum of membership values is 1. This ensures a smooth transition between the fuzzy sets and better control for the robot’s movement.

This picture represents the ascending slope of the medium membership function and the disturbance having a value between small and large. It creates two similar right triangles, ABC which is formed by the full slope and AMN which is the smaller triangle formed by the membership value of the disturbance.

This means that MN/BC is equal to AM/AB, so the side MN = AM × BC / AB. Since BC is equal to 1, AM is disturbance - small and AB is medium - small, the membership degree of the disturbance (MN) is (disturbance - small)/(medium - small). This geometric reasoning is applied to every ascending and descending slope for each membership function in order to compute every membership value.

3.2.2 Fuzzy Inference

The inference stage applies a set of IF-THEN rules that map each fuzzy set to specific gain multiplier values. These rules encode the control strategy: how aggressively the controller should respond to the magnitude of the disturbance. The inference() method handles the inference and defuzzification stages of the fuzzy control system. The Fuzzy PI algorithm uses three rules, stored in the configuration (the list Rules: [(0.8, 0.7), (1.0, 1.0), (1.2, 1.3)]) where each tuple represents the KP and KI multiplier for the corresponding fuzzy set.

Rule 1: IF the disturbance is small THEN KP×= 0.8 and KI×= 0.7 . This is interpreted as: If the disturbance between the values read by the sensors is small and the robot is near the center then the gains are reduced, KP by 20% and KI by 30%, for smooth and gentle corrections. This prevents the robot from making oscillations around the center position.

Rule 2: IF the disturbance is medium THEN KP×= 1.0 and KI×= 1.0 . This is interpreted as: If the disturbance is medium and the robot is moderately off-center then keep the nominal gains. This is the nominal point of the controller.

Rule 3: IF the disturbance is large THEN KP×= 1.2 and KI×= 1.3 . This is interpreted as: If the disturbance is large and the robot is far off-center then boost the gains, KP by 20% and KI by 30% for aggressive correction. The boosted proportional gain provides immediate corrective force while the boosted integral gain helps eliminate accumulated steady-state error.

3.2.3 Defuzzification

The defuzzification stage is the conversion of the fuzzy set back into a crisp value. The method mentioned previously computes a weighted average defuzzification with the membership values serving as weights. From the inference stage, the controller combines the gain multipliers by aggregating all active rules into a pair of final adaptive gains. The denominator, being the sum of all membership values, is always equal to 1 and ensures that the result is normalized. These adaptive gains are then given to the compute() method to obtain the final turn rate.

3.2.4 Computing the Output

The compute() method combines all stages of the fuzzy logic to obtain the final turn rate the robot must execute. It receives the setPoint which is usually 0, indicating perfect balance between sensors, the measuredPoint which is the actual difference between sensors and the change in time. The disturbance is the difference between setPoint and measuredPoint. Using the adaptive KP and KI computed from the fuzzy system, there are constructed the PI terms of the controller.

The proportional term is the product of the disturbance and the adaptive gain and acts as the correction to the current disturbance, while the integral term is the product of gain and the disturbance multiplied with the change in time, constrained by the integral interval in order to prevent the integral from growing infinitely large.

Output=(KP×Imbalance)+(KI×Imbalance dt)

The output final turn rate is comprised of the two PI terms and restricted to the output interval, so that the motors of the robot don’t receive impossible commands. The method returns the disturbance and the output representing the turn rate.

The reset() method is used to clear the integral accumulator, and it’s called whenever the controller is reinitialized. For example, after the robot handles a lane crossing event.

The advantage of this approach over a fixed-gain PID controller is that the Fuzzy PI controller automatically adjusts its output according to its position from the center. It weakens when the robot is close to the center to avoid jitters and strengthens the output when the robot is far from the center to avoid sluggish reaction to large deviations. In this behavior, the fuzzy logic provides a smooth transition between control strategies.

3.3 Fuzzy Rule-Based Algorithm

This algorithm relies only on fuzzy logic to control the movement of the robot based on disturbance created by the difference between values read from the sensors. Unlike the Fuzzy PI algorithm, which only tunes the gains of the PI controller, the value of disturbance and the derivative of disturbance are fuzzified and mapped through a complete Mamdani type fuzzy inference system directly to the turn rate given to the motor.

This approach has two advantages. The first one is the control strategy is expressed in natural language rules rather than mathematical gain values. This makes it easier to tune the boundaries of the fuzzy sets. The second one is by using the current disturbance and the rate of change (trend) the controller is able to anticipate future states. The correction is weakened when the robot is recovering to the center and it is strengthened when the robot begins to deviate from the center.

The algorithm is implemented in the FuzzyRuleBased class and comprises of the four stages of the Mamdani fuzzy controller: fuzzification of the disturbance and the trend, fuzzy inference through a set of 15 rules, aggregation of the active conclusion rules and defuzzification to obtain the crisp final turn rate.

The FuzzyRuleBased begins with the constructor, initializing from the configuration the fuzzy sets for disturbance, derivative of disturbance and the output, the interval ends restricting the output and the fuzzy rules. The interval ends are necessary to prevent the motor of the robot from executing impossible commands. The previous disturbance is set to 0 to compute the derivative. The variables first call, last active rules and previous active rules keep track of which rules are firing and by how much to debug easier. The output singletons define the magnitude of the turn rate.

The clamp method is a helper function which limits the range of the membership value of each fuzzy set to be between 0 and 1.

3.3.1 Fuzzification of Disturbance

The fuzzifyDisturbance() method takes the crisp value of the disturbance, as the first input, and computes the membership degree for each fuzzy set: negative large NL, negative small NS, zero ZE, positive small PS and positive large PL. The µNL, µNS, µZE, µPS and µPL represents the membership degrees, with values between 0 and 1 and describe how much the input resembles each set simultaneously. The reason these fuzzy sets are defined on signed disturbances, unlike the Fuzzy PI algorithm, is that this allows the controller to distinguish between left and right deviation.

These five fuzzy sets are defined by triangular and trapezoidal membership functions. They are delimited by the boundary and signed values from the configuration: -S=-3.0, -M=-6.0,  +S=3.0, +M=6.0. These boundaries determine where one fuzzy set begins and another ends, with overlapping regions where the disturbance partially belongs to two sets at the same time.  It checks for each fuzzy set because there can be more than one non-zero membership value, and that’s why the method returns all five membership values.

The picture above presents graphically the intervals where the disturbance can be situated and what membership value can have, describing a triangular/trapezoidal transition. The disturbance grows or is reduced resulting in smaller membership function for the initial set and a higher membership function for the next set.

For negative large, a trapezoidal membership function, has full membership (µNL = 1.0) when d ≤ -M. Then it decreases linearly from 1 to 0 as the disturbance increases from -M to -S. Zero membership (µNL = 0.0) when d > -S. This represents a strong drift to the left.

For negative small, a triangular membership function, has zero membership (µNS = 0.0) when d < -M or d > 0. It increases linearly from 0 to 1 as the disturbance increases from -M to -S until it reaches its peak when the disturbance is exactly -S and has full membership (µNS = 1.0). Then it decreases linearly from 1 to 0 as the disturbance increases from -S to 0. This represents a slight drift to the left.

For zero, a triangular membership function, has zero membership (µZE = 0.0) when              d < -S or d > +S. It increases linearly from 0 to 1 as the disturbance increases from -S to 0 until it reaches its peak when the disturbance is exactly 0 and has full membership (µZE = 1.0). Then it decreases linearly from 1 to 0 as the disturbance increases from 0 to +S. This represents the robot being approximately centered in the lane.

For positive small, a triangular membership function, has zero membership (µPS = 0.0) when d < 0 or d > +M. It increases linearly from 0 to 1 as the disturbance increases from 0 to +S until it reaches its peak when the disturbance is exactly S and has full membership (µPS = 1.0). Then it decreases linearly from 1 to 0 as the disturbance increases from +S to +M. This represents a slight drift to the right.

For positive large, a trapezoidal membership function, has full membership (µPL = 1.0) when d ≥ M. It decreases linearly from 1 to 0 as the disturbance decreases from +M to +S. Zero membership (µPL = 0.0) when d < +M.

This picture represents the ascending slope of the medium membership function and the disturbance having a value between small and large. It creates two similar right triangles, ABC which is formed by the full slope and AMN which is the smaller triangle formed by the membership value of the disturbance.

This means that MN/BC is equal to AM/AB, so the side MN = AM × BC / AB. Since BC is equal to 1, AM is disturbance - small and AB is medium - small, the membership degree of the disturbance (MN) is (disturbance - small)/(medium - small). This geometric reasoning is applied to every ascending and descending slope for each membership function in order to compute every membership value, as was proved for the Fuzzy PI algorithm. The trapezoidal functions have only one slope and the membership values are implemented in the code with the help from the clamp() method to restrict the results to the interval [0, 1], while the triangular functions have two mirrored slopes.

3.3.2 Fuzzification of the Trend (Δd)

The second input for the fuzzy control system is how the disturbance is changing between consecutive control cycles. It is computed as Δd= dcurrent - dprevious. The physical interpretation of the trend depends on its sign. A positive trend means the new disturbance is higher than the previous one, so the robot is moving further from the center while a negative trend means the new disturbance is lower than the previous one, so the robot is recovering towards the center.

The fuzzifyDelta() method takes the crisp value of trend of disturbance. It computes the membership degree for each fuzzy set: negative N, zero Z, and positive P. The µN, µZ and µP represents the membership degrees, with values between 0 and 1 and describe how much the input resembles each set simultaneously.

These three fuzzy sets are defined by triangular and trapezoidal membership functions. They are delimited by the threshold value from the configuration: D=3.0.

The picture above presents graphically the intervals where the trend of disturbance can be situated and what membership value can have, describing a triangular/trapezoidal transition.

For negative, a trapezoidal function, has full membership when Δd≤ -D and decreases linearly to 0 at Δd=0. This represents the fact that the disturbance tends to decline and the robot recovers to the center.

For zero, a triangular function, has full membership at the peak Δd=0 and decreases linearly to the base from -D to +D. This represents a stable disturbance with little change.

For positive, a trapezoidal function, has full membership when Δd≥ +D and decreases linearly to 0 at Δd=0. This represents the fact that the disturbance tends to grow and the robot furthers away from the center.

3.3.3 Fuzzy Inference Rules

The core of the Fuzzy Rule-Based controller is a set of 15 IF-THEN rules (five fuzzy sets for the disturbance and three for the trend) that map every combination of disturbance and trend to a final output turn rate. The output is expressed as seven fuzzy sets (negative large NL = -50, negative medium NM, negative NS, zero ZE, positive small PS, positive medium PM and positive large PL). Each set is associated with a singleton value that represents the magnitude of turn rate in degrees per second in the table below:

| NL | NM | NS | ZE | PS | PM | PL |
| --- | --- | --- | --- | --- | --- | --- |
| -50 | -30 | -10 | 0 | +10 | +30 | +50 |

Negative output values command the robot to turn left, positive values command the robot to turn right and zero commands the robot to continue straight. The singleton values were chosen to express the range of the EV3 DriveBase, from gentle to aggressive corrections, while remaining below the emergency turn speed of 100 deg/s.

The 15 rules are put in a 5×3 matrix, whose rows correspond to the five disturbance fuzzy sets and columns to the three trend of disturbance fuzzy sets:

| Disturbance\Trend | N | Z | P |
| --- | --- | --- | --- |
| NL | PM | PL | PL |
| NS | PS | PM | PM |
| ZE | ZE | ZE | ZE |
| PS | NS | NM | NM |
| PL | NM | NL | NL |

The expanded rule table, encoded in the configuration as a list of tuples (disturbanceIndex, deltaIndex, outputIndex) and loaded into memory at the constructor express the IF-THEN rules and in the table below they are presented along with their physical meaning:

| Rule | IF disturbance is | AND trend is | THEN turn rate is | Meaning |
| --- | --- | --- | --- | --- |
| R1 | NL (-6) | N (-3) | PM (+30) | Far left & recovering => moderate right turn |
| R2 | NL (-6) | Z (0) | PL (+50) | Far left & stable => strong right turn |
| R3 | NL (-6) | P (+3) | PL (+50) | Far left & worsening => strong right turn |
| R4 | NS (-3) | N (-3) | PS (+10) | Slightly left & recovering => gentle right turn |
| R5 | NS (-3) | Z (0) | PM (+30) | Slightly left & stable => moderate right turn |
| R6 | NS (-3) | P (+3) | PM (+30) | Slightly left & worsening => moderate right turn |
| R7 | ZE (0) | N (-3) | ZE (0) | Centered & recovering => go straight |
| R8 | ZE (0) | Z (0) | ZE (0) | Centered & stable => go straight |
| R9 | ZE (0) | P (+3) | ZE (0) | Centered & worsening => go straight |
| R10 | PS (+3) | N (-3) | NS (-10) | Slightly right & recovering => gentle left turn |
| R11 | PS (+3) | Z (0) | NM (-30) | Slightly right & stable => moderate left turn |
| R12 | PS (+3) | P (+3) | NM (-30) | Slightly right & worsening => moderate left turn |
| R13 | PL (+6) | N (-3) | NM (-30) | Far right & recovering => moderate left turn |
| R14 | PL (+6) | Z (0) | NL (-50) | Far right & stable => strong left turn |
| R15 | PL (+6) | P (+3) | NL (-50) | Far right & worsening => strong left turn |

The rule table has as antisymmetric structure in the case where the disturbance is stable: the output for (NL, Z) is PL (+50), while the output for (PL, Z) is NM (-50). It is equal in magnitude but opposite in sign. Similarly, (NS, Z) maps to PM (+30) and (PS, Z) maps to NM (-30). This symmetry ensures that the robot responds the same to left and right deviations.

The trend dictates the correction strength based on the situation being improved or worsened. When the trend is N (recovering) the magnitude of the correction is reduced and when the trend is P (worsening) the magnitude of the correction is increased. If the trend is Z, meaning the robot isn’t recovering yet, then the magnitude of the correction remains increased.

The centered region, when the disturbance is ZE, maps every trend to ZE keeping the robot going straight regardless of previous changes in the disturbance.

3.3.4 Inference and Aggregation

During each control cycle, the fuzzification stage produces up to two non-zero membership values for the disturbance from adjacent sets and up to two non-zero values for the trend. This means that at any moment, 1, 2 or 4 rules can be simultaneously active, depending on whether both inputs are exactly on a set peak (1 rule), on a transition between two sets for one input and the other is on a set peak (2 rules) or both inputs are on transitions (4 rules).

The inference stage, implemented in the inference() method, evaluates each of the 15 rules by computing the degree activation of the rule. The degree activation is computed by the fuzzy MIN operator, being the minimum of the two input membership values.

If one input has zero membership for a rule’s premise, the rule doesn’t activate and it’s skipped. The method keeps track of the active rules and their strength in the lastActiveRules list for debugging.

In the case of multiple active rules map to the same singleton output, their strength must be combined. The aggregation stage uses the fuzzy MAX operator in the way that for each output singleton, only the highest activation degree among all rules that activate it is kept. This is implemented as an array of the size corresponding to the number of singleton outputs and each entry holds the maximum degree activation for all the rules targeting that singleton.

3.3.5 Defuzzification

The defuzzification stage converts the aggregated fuzzy output into the crisp turn rate value. Since the output are represented as singletons, the defuzzification use weighted average:

turn rate =(aggregation[i] × singleton[i]) / (aggregation[i])

The sum collects each output singleton. Singletons with zero aggregated strengths don’t contribute to the turn rate and if no rules are active the default output is zero meaning driving straight.

3.3.6 Computing the Output

The compute() method combines all stages of the fuzzy logic to obtain the final turn rate the robot must execute. It receives the setPoint which is usually 0, indicating perfect balance between sensors, the measuredPoint which is the actual difference between sensors and the change in time.

The membership values of the disturbance and delta are retrieved from the fuzzify() methods. Based on them, the rules are evaluated to find out which are activated in the inference() method. The output turn rate is defuzzified from the aggregated fuzzy sets and is clamped so that the robot doesn’t receive commands that isn’t able to execute.

4. The Maze Solver Application

This project also implements a maze-solving algorithm in which the Lego Mindstorms EV3 navigates through a maze transposed as a matrix, being discovered at runtime. The maze is composed of cells separated by walls and the robot explores every possible cell while finding a path from the starting point to the finish. The robot uses four sensors to detect its environment: an ultrasonic sensor in the front to detect the obstacles ahead, two ultrasonic sensors on the left and on the right side to balance itself in corridors and to detect open passages, and a gyroscope sensor to maintain accurate heading after turns. The forward sensor is put on a motor that rotates 90 degrees left and right to allow the robot to find the possible paths when facing a wall in the front, if there are any.

The robot drives through the maze using a Depth First Search (DFS) algorithm that maintains a map of the explored paths in memory. The robot takes each cell as a node in a graph, records visited cells and which directions are open, then explores unvisited neighbors before backtracking. This guarantees complete exploration of any connected maze. The robot stops at each junction and scans for open passages in three directions: forward, left and right. It consults the map to determine which adjacent cells are unvisited and then advances into unvisited cells or backtracks along its path to find undiscovered branches. Between junctions, the robot uses control algorithms: PID, Fuzzy PI or Fuzzy Rule-Based to regulate its forward speed based on the distance to the next wall in the front.

The architecture of this application extends the same Object-Oriented, SOLID-compliant structure from the lane keeping application. The initiliazeConfiguration() method provides all necessary variables for the maze solver task.

The three ultrasonic sensors and the gyroscope sensor are assigned to their respective ports, along with the motor for the forward sensor. CellDistanceNS and CellDistanceEW represent the length and width of each cell in the maze. OpenPassageThreshold is the minimum value the sideways sensors detect in order to consider it an open path. The ObstacleThreshold is the distance from the wall in front so that the robot stops and checks for alternative paths. TargetForwardDistance is the setpoint for the control of speed algorithm. BaseSpeed is the speed of the robot while driving in a long “corridor” from one cell to another, while MinSpeed is the minimum forward velocity to prevent the robot from taking too much time to traverse a cell. The CurveSpeed is forward velocity during a 90 degrees curve turn and the CurveTurnRate is the amount of degrees to turn in one second. BackupDistance is the distance the robot reverses in order to execute a turn in a junction. The HeadingThreshold and CenteringThreshold define tolerance zones for errors smaller than three degrees from the fixed 90 degrees turn or twenty millimeters distance from the left or right wall. The CenteringGain is the conversion of the lateral disturbance (left distance - right distance) into a corrective turn angle, capped at ±15 degrees to prevent oscillation.

4.1 Maze Map

The MazeMap class is the data structure that gives orientation and position to the robot in the maze. It keeps track of the explored cells of the maze and implements the Depth-First Search navigation logic. The class uses a coordinate system based on a compass with cardinal directions encoded as integers. The DX and DY tuples define the displacement on the x and y axis when the robot changes its position. Moving North (forward) increases y by 1, moving East (right) increases x by 1, moving South (backward) decreases y by 1 and moving West (left) decreases x by 1. The TURN tuple contains the rotation angles in order to face each heading direction: 0 for forward, +90 for right, 180 for backward and -90 for left. The DFS list contains the priority order of the directions the robot takes and the BACKDIR dictionary provides the reverse lookup having as key the coordinate displacement and as value the corresponding direction.

The constructor initializes the starting position of the robot with the heading towards North, an empty set of visited cells and the empty path stack, which will store the sequence of cells the robot has passed through. With this, the robot can backtrack its steps once it reaches a dead end and avoid repeating the path over and over again.

The markVisited() method adds the current cell to the visited cell and the gyroTarget() method returns the expected gyroscope reading for the current heading. The planMove() method is the decision-making function. It receives as parameter the set of directions that are physically open (they are not walls) at the current junction. By iterating the DFS priority order, it checks for directions that are physically open and the cell in that direction wasn’t visited. The first direction that satisfies both conditions is returned along with the required turn angle. If there aren’t any unvisited and open directions, the method returns None and the backtracking method will be required.

The DFS priority order is defined as (0, 3, 1, 2) which translates to Forward, Left, Right and Backward. This means that the robot will continue straight until reaches a wall. It will turn left, if possible, otherwise will turn right. Finally, the robot turns around only if it’s situated at a dead end.

The planBacktrack() method handles the situation where the robot can’t navigate to other unvisited cells. It looks at the top of the path stack to retrieve the previous cell, computes the direction needed return to using the BACKDIR dictionary and calculates the required turn angle. If the stack is empty, the method returns None, meaning the entire maze has been explored. The applyTurn() method updates the heading before the physical turn and with commitMove() method pushes the current position onto the path stack, sets the coordinates in the heading direction and marks the new cell as visited. When backtracking, the commitBacktrack() method pops the previous position from the path stack to restore the coordinates where the robot came from.

4.2 Maze Solver Controller

The MazeSolverController class implements the Controller interface and executes the DFS algorithm for the robot to traverse the maze. It handles the sensors reading, motors turning, headings aligning, centering in corridors and precise driving for distances from cell to cell.

The constructor takes the inherited attributes from the super abstract class Controller and adds the specific hardware for the maze: the three ultrasonic sensors, the gyroscope and the motor sensor. It instantiates the MazeMap object for the orientation and positioning of the robot and stores the configuration parameters for driving, turning and correction tasks.

The scanForward() method reads the distance from the forward ultrasonic sensor. Due to the fact that ultrasonic sensors are susceptible to spike noise caused by the echoed signals bouncing off the angled walls, the method takes three consecutive readings, sorts them and returns the median. This way, spikes are ignored.

The scanJunction() method determines which directions are open in the current cell. It receives the values read by the sensors and compares them with the openPassage threshold. Each value that exceeds the threshold is added to the set of open directions. The method returns this set to be passed to the MazeMap’s planMove() method for decision making.

The alignHeading() method corrects the physical orientation of the robot to match its intended compass heading using the gyroscope. Mechanical errors such as wheel slippage can cause a deviation of the actual heading of the robot from the target, and after many turns, those accumulated errors will drive the robot into a wall. Hence, the need of this method to keep the robot from misaligning with the maze grid.

The method computes the angular error between the gyro target and the current gyro reading. Then the error is normalized to the [-180o, 1800] interval so that the robot takes the angle doesn’t exceed more than one turn. If this needed correction is more than three degrees, then the robot executes a fixed turn of the computed angle and waits 200 ms for the mechanical system to settle. Otherwise, the alignment is skipped because such small corrections are within the sensor’s noise.

The centerInCorridor() method adjusts the lateral position of the robot within the corridor to prevent it from driving into a side wall, using the difference between the left and right distance from the walls. If both walls are there (there is no T junction) and the difference is relevantly high, the robot executes a small corrective turn. The correction angle is computed as difference × centeringGain, where this gain is 0.1 deg/mm, meaning a 100 mm offset produces a 10o correction.

The cellDist() method returns the physical distance the robot must traverse to cross one cell in the given absolute direction. Since the maze cells are rectangular, the distance depends on whether the robot is moving along the North-South axis or the East-West axis. For the North and South directions were used even indices and odd indices were used for the East and West directions.

The executeTurn() method performs the physical rotation of the robot by handling three cases.

The first case, the turn angle is  0o. This means that the robot is already facing the correct direction and no action is needed.

The second case, the turn angle is ±90o. The method reads the starting angle from the gyro, computes the target angle and drives a smooth curve at the configured curve speed and curve turn rate while polling the gyro in a tight loop until the target angle is reached. This produces a gentle arc which reduces wheel slippage on smooth surfaces, as the maze has.

The third case treats turn-arounds where the method uses the DriveBase’s robot.turn() method which uses the motor encoders rather than the gyro. This is because at a full 180o turn at the slow curve rate would take too long.